#define BLYNK_TEMPLATE_ID "TMPL6vSyFiqSG"

#define BLYNK_TEMPLATE_NAME "MAX30100 Pulse Oximeter"

#define BLYNK_AUTH_TOKEN "8pr62XDYST9Gq574GY-uSc3VKrukKPtb"

#include <Wire.h>

#include "MAX30100_PulseOximeter.h"

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

char ssid[] = "Tplink";

char pass[] = "0187754ss@@";

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

#define OLED_RESET -1

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

#define REPORTING_PERIOD_MS 1000

PulseOximeter pox;

uint32_t tsLastReport = 0;

void onBeatDetected() {

Serial.println("Beat!");

}

void setup() {

Serial.begin(115200);

// Connect to WiFi and Blynk

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// Initialize OLED

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println(F("SSD1306 allocation failed"));

while (true);

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 0);

display.println("Initializing...");

display.display();

// Initialize MAX30100

if (!pox.begin()) {

Serial.println("MAX30100 FAILED");

display.setCursor(0, 10);

display.println("MAX30100 FAILED");

display.display();

while (true);

} else {

Serial.println("MAX30100 SUCCESS");

display.setCursor(0, 10);

display.println("MAX30100 SUCCESS");

display.display();

}

pox.setIRLedCurrent(MAX30100_LED_CURR_7_6MA);

pox.setOnBeatDetectedCallback(onBeatDetected);

}

void loop() {

Blynk.run();

pox.update();

if (millis() - tsLastReport > REPORTING_PERIOD_MS) {

float hr = pox.getHeartRate();

float spo2 = pox.getSpO2();

Serial.print("Heart rate: ");

Serial.print(hr);

Serial.print(" bpm / SpO2: ");

Serial.print(spo2);

Serial.println(" %");

// Send to Blynk Cloud

Blynk.virtualWrite(V0, hr);

Blynk.virtualWrite(V1, spo2);

// OLED Display

display.clearDisplay();

display.setCursor(0, 0);

display.setTextSize(1);

display.println("Pulse Oximeter");

display.setTextSize(2);

display.setCursor(0, 20);

display.print("HR: ");

display.print((int)hr);

display.println(" bpm");

display.setCursor(0, 45);

display.print("SpO2: ");

display.print((int)spo2);

display.println(" %");

display.display();

tsLastReport = millis();

}

}